26, July, 1996

Sanjay

SERIAL.DOC

----------

There is a new serial driver in the DEVDRVRS\SERIAL directory.

This directory contains code for using SCC2, SCC3 and SCC4 of the

MC68360 QUICC as ASYNC UART's.

This code was developed for IPX dialout (AsyncGateway) support.

Any app that requires a UART interface can use this code. The

interface is as below. Only a general explanation is given. For

details, see the function prototypes and the IPX dialout code

for how the interface is used. Especially note the way buffers

are returned to the respective layers thro' the use of callback

functions (post routines).

NOTE: There is no idle timeout if no packet data reception

or transmission occurs. Implement such things at the app level.

BAUD DIVISORS ARE PROCESSOR SPEED DEPENDENT. HERE WE USE

DIVISORS ASSUMING 25MHz clock speed. If 33MHz is used, then

different divisors are needed.

Initialization

---------------

Call the function "setup_serial_port_for_use() to init the interface

to a specific SCC and begin using it.

Packet Reception

----------------

Received packets are passed to a callback function that is provided

during the init phase as a parameter to the init function

Packet Trasmission

------------------

The function serial_tx_packet() can be called to send a packet.

The buffer provided for send should only be freed in the

tx_complete function that is provided as a parameter to the

init function.

De-Init

-------

Call the function free_serial_port_from_use() to disable all transmit's

and receives on a specific channel and release the port from the serial

driver.

Others

------

1. To be able to associate a particular app session or table entry

when a packet is received (which can occur at any time), a pointer

to the session or entry is passed as a parameter during init time.

This same parameter is returned to the app when a packet is received

at its packet receive callback function as a parameter.

2. Receive buffers should be passed back to the driver after use by

calling the receive complete function that is provided as a

parameter to the receive callback routine.

3. The receive buffers reserve some space (currently 128) bytes

ahead of the point where the actual data is received. This is

for the convenience of apps that want to build network headers in

the same buffer for transmission on the network.

Note that when the app is called with a receive buffer, the

pointer points to actual data (just after this 128 byte header)

space. Also when the app returns the buffer, it should return

the pointer pointing to the same place (at start of data).

To be able to find out the descriptor assoicated with a receive

buffer, the pointer to the descriptor is stored at the start of

the receive buffer (1st few bytes is a pointer to the descriptor).

We will need this when app calls the receive complete routine.

The app will have only pointer to the received buffer.

Functions Provided

------------------

See SERIAL.H for exported functions from this module. This header is

in the global INCLUDE directory.