MTS2EA   adapter cable for a modem

9 PIN MALE (ip adapter)
25 PIN MALE (modem)

1 not connected

2 (rcv) ----------------------------------
2 (xmt)

3 (xmt) ---------------------------------   3 (rvc)

4 (dtr) ----------------------------------   8 (cd)

5 (gnd) ----------------------------------  7 (gnd)

6 (dsr) ----------------------------------- 20 (dtr)

7 (rts) ------------------------------------ 5 (cts)

8 (cts) ------------------------------------ 4 (rts)

9 not connected  

9 PIN MALE (ip adapter)
9 PIN MALE (modem)

1 not connected

2 (rcv) ----------------------------------
3 (xmt)

3 (xmt) ---------------------------------    2  (rcv)

4 (dtr) ----------------------------------   1 (cd)

5 (gnd) ----------------------------------  5 (gnd)

6 (dsr) ----------------------------------- 4 (dtr)

7 (rts) ------------------------------------ 8 (cts)

8 (cts) ------------------------------------ 7 (rts)

9 not connected  

