E&M Loop Back Test

Voip A 

Ch 1 FXS and Ch 2 E&M, type II, 4 wire, and dial tone. Make sure pass thru is disabled. 

Inbound of 101 ch 1 and 102 ch 2. 

Outbound of destination pattern 201, total digits 3, IP address of voip B, and H.323 protocol. 
Outbound of destination pattern 202, total digits 3, IP address of voip B, and H.323 protocol. 

Voip B 

Ch 1 FXS and Ch 2 E&M, type II, 4 wire, and dial tone. Make sure pass thru is disabled. 

Inbound of 201 ch 1 and 202 ch 2. 

Outbound of destination pattern 101, total digits 3, IP address of voip B, and H.323 protocol. 

Outbound of destination pattern 102, total digits 3, IP address of voip B, and H.323 protocol. 

You would need to attach a regular telephone set on ch 1 of both voips and a "back to back" cable between ch 2 E&M of both units. 

The dialing procedure is as follow. Pick up the telephone on ch 1 of voip A, get a local dial tone, dial 202, call goes to remote channel 2 E&M and loops back to ch 2 E&M of voip A(102), get local dial tone again, dial 201, and the telephone on ch 1 FXS of the remote rings. This will test the E&M ports. 

The "back to back" cable is made with the following pinouts. 

1 --- 2 

2 --- 1 

3 --- 4 

4 --- 3 

5 --- 6 

6 --- 5 

7 --- 8 

8 --- 7
