The command to check the status of the GPRS connection is the "AT#VSTATE" command.   If the GPRS connection is up, it should respond with #STATE: "CONNECTED".

#VSTATE Display Current Status

Definition: This command directs the TCP/IP stack to display the current status of the device.

Command

Possible Responses

AT#VSTATE

#STATE: IDLE


Note: Idle state.

AT#VSTATE

#STATE: DIALING


Note: Dialing the ISP. Not yet connected.

AT#VSTATE

#STATE: AUTHENTICATING
Note: Connection. Not yet PPP negotiated (PPP OK message).

AT#VSTATE

#STATE: CONNECTED

Note: Connected to Internet. An IP address has been attributed to the TCP/IP stack.

AT#VSTATE

#NO SERVICE


Note: No radio network connection made.

AT#VSTATE

#STATE: DISCONNECTING
Note: In the process of disconnecting from the current communication process.

AT#VSTATE

#STATE: CHECKING

Note: Either, in the process of setting up an outgoing communication session over GSM or GPRS, or in the process of answering an incoming call (manually or automatically) over GSM.
