How to program Ethernet CDMA Wireless modem (MTCBA-C-EN) using Wavecom WPST software:

Hardware and Software requirements:

· PC with Windows 2000 or XP

· Ethernet CDMA wireless modem (MTCBA-C-EN)

· Wavecom WPST software (must be version 1.4 and above)

ftp://ftp.multitech.com/wireless/wpst/WPST_1.4.zip
ftp://ftp.multitech.com/wireless/wpst/WPST_UPDATE_1.4.0.3-UPD1.exe
· Multi-Tech WinMCSI virtual com port software (must be version 1.05 and above)

ftp://ftp.multitech.com/embedded/socketethernetip/mcsi2000_105.exe
Assume IP address for Ethernet CDMA modem = 192.168.2.1

Assume IP address for Windows PC = 192.168.2.2

How to setup Multi-Tech Ethernet CDMA Wireless mode:

· Connect Ethernet port to the Windows PC Ethernet port

· Make sure Windows PC can ping the CDMA modem at IP address 192.168.2.1

· At the windows DOS prompt, telnet to the CDMA modem on port 23 and issue these commands

· admin (default login username) 

· admin (default username and password)

· set ppp ppp0 status disable

· set serial s0 flow-control none

· set ip auto-dialout-raw-mode enable

· set ip auto-dialout-port 5000

· set ip auto-dialout-login disable

· save

· close telnet session

· Wait until CDMA modem is back up (IP led should be blinking) and telnet to the modem on port 5000 and make sure it responds back with AT command.   Close telnet session when done.

How to setup Muti-Tech WinMCSI virtual com port software:

· Install the Multi-Tech WinMCSI software on Windows 2000 or XP machine

· Add a new virtual com port and configure the following

· Direct mode

· Server name / IP address = 192.168.2.1

· No authentication

· Raw mode

· Port No 5000

How to setup Wavecom WPST software:

· Install the Wavecom WPST software on Windows 2000 or XP machine

· Install patch “WPST_UPDATE_1.4.0.3-UPD1.exe”

· Change Windows registry parameter to disable autobaud

HKEY_CURRENT_USER\Software\Wavecom\CDMA Server\Settings
 

Disable the autobaud feature: Autobaud = 0 

· Run WPST program and you should see the virtual com port with the CDMA modem information.

· Refer to WPST software user guide for more information on how to use the WPST software

ftp://ftp.multitech.com/wireless/wpst/WPST_Quick_Start_Guide.pdf
· After the WPST software is done programming the CDMA modem, you MUST press the reset button on the back of the CDMA modem so all the settings can be set back to factory default.

